Fixing JVM Error on Blackberry Devices

If or when you get a JVM Error 102/Reset message, do the following:
1. Connect your BB to the computer.
2. Run JL_Cmder
3. Choose option 5, wipe the device. This will delete all files - OS and data - from your BB. The BB screen will show JVM Error 507. This means the BB has no files installed.
4. Using Desktop Manager, reinstall your software. When you run the Application Loader, check all the available boxes. 
5. When this process completes, restore your most recent backup file.
You should be back to normal, with the exception of having to reregister OTA or other 3rd party apps. Save their reg numbers in your Memo app so that you have them handy.

__________________________________________________

The following table lists errors that can occur in the Java virtual machine (JVM) on BlackBerry handhelds.

Error	Description

101 	Internal JVM error.

102 	Invalid code in file system. The .cod files in the handheld have been checked for modification and it has been determined that there is a problem with one or more .cod files.

103 	The starting address for the boot .cod file cannot be found. This might mean that a boot .cod file has not been installed on the handheld, or that its format is invalid or corrupt.

104 	An uncaught Java exception was thrown in the Java code and diagnosed by the JVM. Execution can be continued or the handheld can be attached to a debugger on a desktop through a serial or USB cable. The event log should contain the traceback of the thrown exception.

105 	An OS file system API returned an error status for a certain operation. This can indicate a corrupt file system or an error in the JVM.

106 	An error has been detected in the graphics system of the handheld.

107 	Internal JVM error.

108 	Internal JVM error.

109 	Internal OS error.

110 	Non-idle event downtime error. A problem has been detected in the accumulation of JVM down time that represents how long the JVM has been idle. This indicates an error in either the OS code or the JVM code.

200 	Application manager threw an uncaught exception. The application manager event thread threw an uncaught exception and so cannot continue running.

201 	Initialization of the cryptographic system failed and the handheld cannot continue to operate.

202 	An attack on the key store has been detected, and the handheld cannot continue to operate.

203 	The application manager console process, usually the Home screen ribbon, has failed, like due to an uncaught exception.

501 	Internal error.

502 	All processes exited. The last Java process has terminated, and there is nothing left to execute.

503 	Internal error.

504 	Internal error.

505 	Internal error.

506 	An uncaught Java exception was thrown in the initial VM Java thread thus killing the only live thread in the system. The event log contains the traceback for the exception.

507 	A dependency on a .cod file could not be satisfied due to a missing .cod file. Load the missing .cod file onto the handheld.

508 	Invalid object. A problem has been detected with a debugger command to the VM.

516 	Error occurred during garbage collection, which might indicate a corrupted file system.

510 	All threads are waiting on objects, which results in a deadlock. The system cannot recover from this state since no thread can release a lock.

511 	A problem has occurred during debugging.

515 	The reachable objects form a group that cannot be represented properly by the VM because there are too many objects or the total size of the objects is too large.

516 	When committing a persistent object, the VM found that the persistent store id counter has reached its limit. The object was not committed.

517 	An inconsistency has been detected in the VM persistent object store.

518 	Internal error.

519 	Internal error.

520 	Internal error.

521 	Indicates that Object.wait() has been executed by a thread that holds a lock on another object; occurs only in simulator if the JvmDebugWaits application switch.

522 	A thread has acquired two locks on objects in an order that doesn't match the order that previous locks for the two types were acquired, which indicates a future potential deadlock situation; reported only in the simulator when the JvmDebugLocks application switch is set.

523 	A critical Java process has died and the device cannot continue to operate normally.

524 	An object has been marked as recovered by the Low Memory Manager but it was not freed during a garbage collection. This is only checked in the simulator under the control of the JvmDebugLMM application switch.

525 	Bad persistent object. An auto-commit operation during a garbage collection has detected a non-persistent object reachable from the persistent store root. The type of the object has been output into the event log.

526 	The class definition for java.lang.Object cannot be found.

527 	The class definition for java.lang.String cannot be found.

528 	The file system is corrupted. Data on the handheld is unrecoverable.

529 	The file system is corrupted. An attempt is going to be made to recover data, but some data might be lost.

530 	Internal JVM error.

531 	Flash memory has been exhausted.

532 	A JVM assertion has been violated. This error can occur only in the simulator, not on an actual handheld.